[image: image1.jpg]


                                                                      Baxi S.p.A.
                                                                      36061 Bassano del Grappa (VI)
                                                                       Italy
                                                                      Via Trozzetti, 20
                                                                       Tel: +39 0424 517111
code 5331810 -  GAS VALVE VR 4601 QB 2019
[image: image14.jpg]Z\
BAXI




Characteristics :  Gas component
Technical characteristics: 
Model : VR 4601 QB
Maximum working pressure : 60mbar

Ambient operating temperature : 0°C ÷ +60°C

Class solenoid : B+J

Nominal tension : 230 Vac

Frequency : 50 Hz

Material : pressure die cast + electric coil + mechanical component
Weight : 1.231 Kg   ( Net. 1.121 Kg )[image: image2][image: image3][image: image4][image: image5][image: image6][image: image7][image: image8][image: image9][image: image10][image: image11][image: image12][image: image13]
