[image: image1]                                                                      Baxi S.p.A.
                                                                      36061 Bassano del Grappa (VI)
                                                                       Italy
                                                                      Via Trozzetti, 20
                                                                       Tel: +39 0424 517111
code 5408560 - O-RING 8,73x1,78 
 Characteristics : Sealing O-ring 
Technical characteristics :
[image: image13.jpg]Z\
BAXI







Dimension : D= 8.73 mm ( ± 0.17 mm )



      d= 1.78 mm ( ± 0.08 mm )




        Hardness shore= 70 ( +/- 5 )




        Range application= -50°C + 150°C



[image: image14.png]







         Tollerances applicable to the burrs





          h-max burr height : 0.13 mm





          p-burr max depth  : 0.08 mm
Note : The O-ring must be seated in their seats , in flat position , no twisted or pinched.


The O-ring in EPDM are not suitable for gas and petroleum products.

They must not be contaminated by the same
Material : EPDM
Weight :   0.009 Kg  (net. 0.001 Kg )
[image: image15.png]N
N

NN
0//////%//
N /// \

N\
NN

NN
AN

A




[image: image2][image: image3][image: image4][image: image5][image: image6][image: image7][image: image8][image: image9][image: image10][image: image11][image: image12]
